Flash ROM:  •
 Future proof POS system with free frmware updates via MetroSet ®2 software and standard PC

Light Source Visible Laser Diode 650 nm ± 10 nm

Audible Indicators 7 tones or no beep

Visual Indicators Green = ready to scan; Red = good read; Yellow = automatic scanning

Host System Interfaces USB, RS232, Keyboard Wedge, IBM 46xx (RS485), OCIA, Laser Emulation, Light Pen Emulation

Scan Pattern Single scan line

Scan Speed 72 scan lines per second 

Scan Angle Horizontal: 50°

Print Contrast 35% minimum refectance difference

Decode Capabilities

1D: Code 39, Code 93, Code 128, UPC/EAN/JAN, Code 2 of 5, Code 11, Codabar, MSI Plessey, GS1 DataBar, Telepen, Trioptic  PDF: PDF417*, MicroPDF417*      *MS9544 only
